CAP V-SAR

Civil Air Patrol Virtual Search and Rescue Trainer

Concept and Implementation Summary  |  Draft  |  6 September 2026

This is a ten-page summary of the CAP V-SAR implementation plan, version 0.7. Technical detail, the full effects catalogue, integration specifications, risk register and open engineering questions live in that document. This summary is written for leadership review and circulation. Nothing described here has been built; this is a proposal.

# 1. Summary

CAP V-SAR is a low-cost desktop simulator for training Mission Pilots, Observers, Scanners and mission base staff in electronic search - locating 121.5, 243 and 406 MHz distress beacons with the Becker SAR-DF 517 and the RHOTHETA RT-600 replacing it.

It runs on members' own computers. It combines commercial flight simulation, custom instrument panels replicating the DF heads, live human air traffic control, and a mission base map that staff join from home. Every sortie is recorded and replayed against the true beacon position in debrief.

THE CENTRAL IDEA

X-Plane does not simulate distress beacons at all. That is the advantage, not the obstacle. Because the entire signal chain must be synthesized, the instructor controls every effect that makes electronic search difficult - terrain masking, radio horizon, the cone of confusion, multipath false bearings - individually, repeatably, on demand. No real aircraft can be told to produce a textbook multipath error. This one can.

Effort: 45-73 developer-days plus 6-12 days of artwork, across five gated phases. A usable single-seat trainer exists at the end of Phase 2, roughly 20 developer-days in. Cost: per-seat software licences, modest and mostly one-time; volunteer time is the dominant cost. Starting team: four people.

# 2. The problem

Electronic search is a perishable skill that is expensive to practise. A training sortie needs an aircraft, fuel, a crew, a practice beacon and cooperative weather - and it delivers whatever signal conditions happen to exist that day.

The consequence is that crews often meet the hard cases for the first time on an actual mission: a reciprocal bearing chased for twenty minutes, a signal masked by a ridge and read as "no beacon", a 406 needle that only updates every fifty seconds, or a search that cannot be flown at all without negotiating with a busy approach controller. Those are precisely the moments where a trained response matters most, and precisely what we practise least.

| What a simulator addresses well | What it does not |
| --- | --- |
| DF operation and mode selection under time pressure | Aircraft handling and airmanship |
| Interpreting ambiguous, degraded or false signals | Visual scanning for wreckage |
| Search pattern discipline and navigation | Fatigue, turbulence, cockpit noise |
| Crew coordination between MP, MO and MS | Ground team interface and physical clues |
| Mission base reporting and retasking discipline | Regulatory currency requirements |
| Repeating a difficult scenario until the crew gets it right | - |

# 3. What CAP V-SAR is

| Layer | Component | Function |
| --- | --- | --- |
| 1. Simulation | X-Plane 12 plus a small scripted plugin | Flight, terrain, and a synthesized beacon signal with instructor-controlled effects |
| 2. Instruments | Air Manager panels | Becker 517 and RT-600 DF heads; a second COM panel at the Observer station, as in the aircraft |
| 3. Radios | PilotEdge live ATC, plus a CAP mission voice net | Human controllers on one radio, mission base on the other - two radios, two push-to-talk keys |
| 4. Mission base | Browser client and a CalTopo training map | Live aircraft tracking, bearing plotting, retasking, and debrief replay against beacon truth |

Windows, Mac or Linux. Mission base staff install nothing - they join in a browser from home. All participants connect over a private overlay network, so nothing is exposed to the public internet.

# 4. Why it works

The instructor can switch each of the following on independently. This catalogue is, in effect, the syllabus - each entry is a discrete teachable failure that a real sortie can only deliver by chance.

| Effect | What it teaches |
| --- | --- |
| Radio horizon | Deliberate altitude selection; why search altitude matters |
| Terrain masking | Recognising a valley null and repositioning, rather than concluding "no beacon" |
| 406 burst cadence | Patience with a needle that updates roughly every fifty seconds and holds between bursts |
| 121.5 homer | The 406-to-121.5 transition on final approach to the source |
| Cone of confusion | Confirming station passage instead of chasing the needle overhead |
| Multipath reflection | Recognising a confident, wrong bearing. The single most valuable lesson in the set. |
| Wrong 406 channel | Why the Becker 517 hears only two of eighteen channels, and why the RT-600 replaces it |
| Intermittent or fading beacon | Not abandoning a search on signal loss |

The recorded track replayed against ground truth is the product. A real sortie gives you that only partially, and at far greater cost.

# 5. How it would be used

| Mode | Participants | Description | Available |
| --- | --- | --- | --- |
| A - Solo | One trainee, instructor alongside | Single computer. Trainee flies and works the DF; instructor places the beacon and injects effects. | End Phase 2 |
| B - Crew | MP, MO, MS at separate stations | One simulator drives several instrument stations. Crew coordinates on intercom. | Phase 3 |
| C - Networked | Several crews plus remote mission base staff | Multiple simulators, live ATC, mission voice net, shared map. Approximates a full exercise. | Phases 4-5 |

Each mode is usable on its own. If the project stopped after Phase 3 it would remain a worthwhile aircrew trainer.

| Role | Station and function |
| --- | --- |
| Mission Pilot | Simulator with yoke and pedals; flies the pattern and works ATC |
| Mission Observer | DF head and second COM panel; bearing log and reports to base |
| Mission Scanner | Instrument panel and outside view; scan and clock-position discipline |
| Instructor / SET | Places beacons, injects effects and faults, adjudicates, debriefs |
| Mission Base (IC, AOBD, MRO) | Browser client, remote; tracking, plotting, retasking |

# 6. A representative scenario

Scenario C-1 is drawn from a sortie the unit actually flew. A 121.5 MHz ELT is transmitting from the ground at the ESCON intermediate fix on the ILS approach to McClellan-Palomar (KCRQ), where the plate calls for 3,300 ft. There is no satellite alert - 121.5 has not been satellite-monitored since 2009 - so the tasking arrives as reports of a signal in the approach area, with no position.

SoCal Approach protects the 3,300 ft approach altitude for arriving IFR traffic and issues the aircraft block altitudes above and below it. The crew searches the upper block, waits out an arrival passing between the blocks, then takes the lower one, homing on a continuous 121.5 signal with a multipath offset from terrain to the east.

THE TRAINING VALUE IS NOT THE BEACON

A strong signal on flat ground near an airport is not hard to localise. The value is that this search cannot be flown at all without continuous negotiation with a live approach controller, on the final approach course, with the ideal search altitude occupied by traffic. It is the sortie most likely to defeat a crew that has only ever searched over open terrain - and the most common failure is not a DF error but a crew that accepts restrictions passively instead of explaining what it actually needs.

Variants of the same geometry cover a ramp false alert (the most common real outcome), a 406 beacon with burst cadence, two aircraft deconflicted by ATC, and a genuine target in terrain east of the field.

# 7. What a session looks like

A training session is a single sortie, briefed and debriefed like a real one. It fits comfortably into a squadron evening or a weekend morning.

| Phase | Duration | Content |
| --- | --- | --- |
| Brief | 10 min | Instructor issues the tasking as it would arrive in reality - an alert with a position and uncertainty, or a report with neither. Crew assigns roles, plans the search, anticipates airspace constraints. |
| Setup | 5 min | Stations up, both radio networks checked, DF configured and training frequencies set. Mission base opens the map. |
| Sortie | 30-40 min | Departure, coordination with ATC, the search itself, localisation, and the report to base. The instructor injects effects and faults from a separate station and does not intervene otherwise. |
| Debrief | 15-20 min | Beacon truth revealed. The recorded track replayed against it, against the bearings reported, and against any ATC restrictions issued. Decision points examined - particularly the first radio call and any bearing chased in error. |

The same scenario can be reflown immediately with a different beacon position or a different fault, which is the point: the crew that misread a multipath bearing gets to meet one again ten minutes later rather than in eighteen months' time.

# 8. What it trains, and how we would know

Training objectives are written as observable behaviours so they can be mapped to task requirements if credit is pursued: DF configuration and mode selection under time pressure; homing intercepts; recognising reciprocal and multipath bearings; altitude management for radio horizon; working a search in controlled airspace; bearing logs and formatted reports; scan discipline; and, at mission base, refining a search area from accumulating negative information.

A trainer with no measure of effect gets judged on how impressive it looks. These are proposed instead:

| Measure | Method |
| --- | --- |
| Time from first signal to overhead | Recorded automatically; trend across repeated scenarios per crew |
| Track efficiency | Distance flown after acquisition versus straight-line distance |
| False-bearing recovery | Time spent on a wrong bearing before correction |
| Subject-matter expert acceptance | Experienced Mission Pilots rate each effect against reality before any trainee uses it - a mandatory Phase 3 gate |
| Transfer to the aircraft | Longer term: instructor assessment on real practice-beacon sorties, before and after |

# 9. Architecture at a glance

Beacon truth lives on a server, not on the trainee's computer. The simulator computes what the direction finder would see - bearing, line of sight against real terrain, signal strength, transmission cadence - and publishes the result as simulator variables. The instrument panels only display it.

| Flow | Path |
| --- | --- |
| Aircraft position | Simulator → local bridge → tracking service → CalTopo map, mission base client, and the debrief recorder |
| Beacon truth | Scenario service → local bridge → simulator variables → DF engine → instrument panel |
| Voice | Two independent networks: PilotEdge for ATC, a private server for the CAP mission net |

Two consequences worth noting. Trainees cannot discover the beacon position by inspecting files on their own machine, because truth arrives only as runtime values while a scenario is running. And because the propagation model is separate from the instrument faceplates, the Becker 517, the RT-600 and any future head are all driven by one shared implementation.

The only place this system touches a tool also used for real missions is CalTopo. Exercise mapping therefore runs on a separate training account with separate credentials and unmistakable simulated callsigns.

# 10. Delivery roadmap

| Phase | Scope | Exit criterion | Effort |
| --- | --- | --- | --- |
| 0 | Foundation | Our own plugin visibly moves a needle in the instrument software | 3-5 days |
| 1 | Propagation engine | Bearings accurate to within one degree; a terrain null appears where the map says it should | 6-10 days |
| 2 | Becker 517 head, solo mode | A trainee flies a complete homing intercept alone | 8-12 days + art |
| 3 | Effects library, RT-600, crew mode, validation | Instructor reproduces a multipath false bearing on demand; a three-person crew runs a sortie; experienced MPs sign off the effects | 9-13 days + art |
| 4 | Tracking and mapping | A remote observer watches a sortie live on a training map opened from a simulated alert | 9-15 days |
| 5 | Radio net, mission base client, debrief | A full networked exercise with remote base staff, followed by a track-versus-truth debrief | 10-18 days |
| P | Parallel: vendor engagement | A written answer from Laminar Research on a small simulator change; not blocking either way | 2-4 days |

Phases 0-2 deliver standalone training value and should be used before committing to the rest. The demonstration at the end of Phase 2 is the highest-value event in the plan: a working trainer that a Wing leader can sit down and fly will do more for the project's standing than any status report.

# 11. What it costs

| Item | Basis | Order of magnitude |
| --- | --- | --- |
| X-Plane 12 | Per pilot computer, one-time | Tens of dollars; a free demo suffices for development |
| Air Manager | Per instrument computer | Per-seat licence; additional display seats licensed separately |
| PilotEdge (live ATC) | Per person, monthly or annual | About $20/month per pilot; shared-cockpit seats $6-10. Roughly $36/month for a three-person crew. Five-hour free trial. |
| CalTopo team account | Annual, per team | Wing may already hold one; a training sub-team may suffice |
| Flight controls | Per pilot station | Yoke and pedals - the largest hardware item after the computer |
| Computers | Per station | Members' own where they meet the specification; budget only for shared stations |
| Server hosting, voice server, VPN, tooling | - | Negligible or free and open source |
| Developer and artist time | Phases 0-5 | The dominant cost. Volunteer. |

Figures are order-of-magnitude and must be confirmed against current vendor pricing before any funding request. Several are per-seat and scale with the number of stations.

# 12. Who builds it

DO NOT STAND UP THE FULL ORGANISATION YET

The most common way a volunteer technical project fails is a steering group meeting monthly about a product that does not exist. Phases 0-2 need four people: an Executive Sponsor engaging occasionally, a Project Officer who is also the Technical Lead, a developer for the propagation engine, and a graphic artist working in short bursts. Everything below is recruited as the phase that needs it approaches.

| Tier | Roles | When |
| --- | --- | --- |
| Governance | Executive Sponsor; Project Officer; Steering Group meeting at phase gates only | From the start |
| Simulation and instruments | Technical Lead; DF Engine Developer; Instrument Developer; Graphic Artist | Phases 1-3 |
| Network and infrastructure | Services Developer; Mission Base Client Developer; Infrastructure and Comms Officer | Phases 4-5 |
| Content and training | Scenario Author / Curriculum Lead; Exercise Instructors; Skills Evaluation Trainers; Documentation Lead | Phase 3 onward |
| Assurance and liaison | SME Validation Panel; Stan/Eval; Safety; Cadet Protection if applicable; Vendor Liaison; Sustainment Owner | As required |

Three positions would stop the project if vacated, and each needs a named deputy: the DF Engine Developer, because nobody else would understand the propagation model; the Project Officer, because the project would stall quietly and unnoticed; and the Vendor Liaison, because the PilotEdge and Laminar relationships are personal rather than institutional and do not transfer automatically.

Two roles deserve particular attention. The Scenario Author is not a coding role, and has more influence than anyone on whether the trainer produces competent searchers. A Skills Evaluation Trainer should be engaged from Phase 3, well before any credit decision, because a SET signs off tasks for qualification - so their view of what a defensible sign-off requires should shape what the system records, not be retrofitted to it.

# 13. Partners and dependencies

The project depends on four external parties and one internal one. None of the external relationships is speculative; two are already established.

| Party | Role in CAP V-SAR | Status |
| --- | --- | --- |
| PilotEdge | Live human air traffic control across the western United States; coordinated exercise slots with briefed controllers | Existing relationship. Coordinated SAR operations are established practice on the network. |
| Laminar Research (X-Plane) | The simulator. Also a small requested change that would let the Observer transmit on a second radio while the pilot works the first. | Existing relationship; receptive previously. The request completes a capability the simulator already partly has, and the project is not blocked if it is declined. |
| Sim Innovations | Air Manager, the instrument panel platform | Commercial product with a documented interface; no relationship required |
| CalTopo | The mission base map, and a documented interface for aircraft positions and map objects | A team account is required to obtain credentials |
| CA Wing Stan/Eval and NHQ | The training-credit path, if one exists | Not yet engaged. This document is the first step. |

# 14. Principal risks

| Risk | Severity | Mitigation |
| --- | --- | --- |
| Nobody owns the system after delivery | High | Name a sustainment owner before Phase 4, with an agreed monthly commitment; everything in a Wing-controlled repository; at least three instructors including one SET |
| Training credit not granted, reducing participation | Medium | Ask Stan/Eval and NHQ early. The value as informal practice stands regardless. |
| Fidelity criticism from experienced crews | Medium | Position honestly as a procedures and decision trainer; SME validation is a mandatory Phase 3 gate, not optional |
| Artwork never gets done | Medium | Identify the artist in Phase 0; fall back to a functional non-replica panel |
| Simulated data reaching an operational system | Medium | Separate CalTopo training account and credentials; simulated callsigns; exercise identifiers on all records |
| Scope growth into a full mission simulator | Medium | Phase gates; stop and use the product after Phase 2 before continuing |
| Cadet participation in an online environment | Medium | Decide early whether cadets are in scope; if so, design the voice net and messaging to Cadet Protection requirements |

# 15. Decisions required

| Question | Why it matters |
| --- | --- |
| Which aircraft and avionics do our crews actually fly? | Sets the cockpit layout and whether the DF panel stands alone |
| Is any training credit being sought - specifically, may a SET sign off which tasks on the basis of a simulator session? | Determines fidelity, documentation rigour, and what each session must record |
| Who is the Executive Sponsor, and who is the Project Officer? | Both must be named individuals; the Project Officer role cannot be shared |
| Who owns the system after delivery? | The first risk above is managed here or not at all |
| Can a CalTopo training team be provisioned? | Keeps exercise traffic off operational maps |
| Are cadets in scope as participants? | Engages Cadet Protection requirements for the online environment |
| Who does the artwork, and who authors scenarios? | Neither is a coding task; both are unbudgeted without a named person |

# 16. The ask

| # | Request | Detail |
| --- | --- | --- |
| 1 | Endorsement to proceed with Phases 0-2 | On existing personal equipment. No Wing funds required. Deliverable: a working single-seat DF trainer anyone can fly. |
| 2 | Two or three experienced Mission Pilots as validators | To rate each signal effect against their own experience before any trainee sees it |
| 3 | An early conversation with Stan/Eval and NHQ on credit | With a SET present, so fidelity and record-keeping are built to the right standard from the start |
| 4 | A CalTopo training team and a PilotEdge exercise slot | Both are inexpensive and both need someone with authority to ask |
| 5 | Hard questions on the plan | It has been red-teamed once. It now needs review by people who fly these sorties. |

Next step. With endorsement, Phase 0 begins on existing equipment: simulator and instrument software installed, and a proof that the architecture holds. The intent is to bring a working single-seat trainer to whoever wants to fly it, and to take hard questions before the Wing conference rather than after.

Summary of CAP V-SAR Implementation Plan v0.7, 6 September 2026. Sources: FAA charts and CAP beacon-prosecution guidance; Sim Innovations, Laminar Research, PilotEdge and CalTopo documentation. Not an approved CAP program.