# California Wing Repeater Reference

Compiled 2026-09-08 — for review, correction, and circulation by Wing communications staff

Provenance & reliability — please read before relying on this This table was compiled from public radio-monitoring / scanner-hobbyist sources (RadioReference.com), not from a CAP-published frequency list. The national TX/RX pair convention is well corroborated across sources. Individual site PL tones and P25 NAC codes should be treated as "likely correct, needs confirmation" — there is an open RadioReference forum thread of hobbyists still working to pin down accurate California values since the narrowband transition. CAP's authoritative site-by-site list lives in member eServices, not publicly. The purpose of circulating this draft is for Wing communications staff to confirm, correct, or flag entries against that authoritative source.

## Sources

| Source | What it provided |
| --- | --- |
| RadioReference — Civil Air Patrol (California) | Site table below, transcribed from here |
| RadioReference Wiki — Civil Air Patrol | National repeater pair / PL / NAC conventions |
| RadioReference Forums — CA narrowband transition search request | Evidence the CA list isn't fully settled even among hobbyists |
| CAP National HQ — Communications Program | Confirms scale (500+ VHF repeaters, dual analog/P25); no site data |
| California Wing — Locations | Wing unit locations, not repeater sites specifically |

## National repeater pair convention

CAP repeaters nationwide use one of two standard TX/RX pairs, dual-mode (analog FM with a PL/CTCSS tone, or P25 digital using a NAC in place of the tone on the same pair):

| Pair | Repeater TX | Repeater RX (input) |
| --- | --- | --- |
| Primary | 148.1250 MHz | 143.5500 MHz |
| Secondary | 148.1500 MHz | 143.7000 MHz |

## California Wing sites

| Site | Location | Freq (MHz) | PL (analog) | NAC (P25) |
| --- | --- | --- | --- | --- |
| R02 | Mt. Diablo (primary) | 148.150 | 162.2 | 656 |
| R39 | Mt. Diablo (secondary) | 148.125 | 162.2 | 656 |
| R08 | Mt. Tamalpais | 148.150 | 67.0 | 29E |
| R09 | Mt. Vaca | 148.125 | 156.7 | 61F |
| R14 | Ward Peak / Lake Tahoe | 148.150 | 146.2 | 68F |
| R16 | (location not listed) | 148.150 | 136.5 | 555 |
| R26 | Mt. St. Helena | 148.150 | 146.2 | 5B6 |
| R12 | Blue Ridge | 148.150 | 173.8 | 6CA |
| R14 | Saddle Peak | 148.150 | 167.9 | 68F |
| R20 | Mt. Palomar | 148.150 | 141.3 | 585 |
| R24 | Onyx (primary) | 148.150 | 127.3 | 4F9 |
| R61 | Onyx (secondary) | 148.125 | 127.3 | 4F9 |
| R28 | Oat Mountain | 148.150 | 156.7 | 61F |
| R35 | Palm Springs | 148.125 | 97.4 | 3CE |
| R62 | Santiago Peak (primary) | 148.150 | 186.2 | 746 |
| R59 | Santiago Peak (secondary) | 148.125 | 186.2 | 746 |

Note: the two “R14” entries (Ward Peak/Lake Tahoe and Saddle Peak) share a site number in the source table — either a Northern/Southern California sub-list numbering collision, or a transcription issue upstream. Flagged here, not resolved.

## Requested from Wing staff

| # | Ask |
| --- | --- |
| 1 | Confirm or correct each site's PL tone and NAC code against the authoritative eServices list. |
| 2 | Resolve the duplicate "R14" site number above. |
| 3 | Fill in the location for R16, currently unlisted in the public source. |
| 4 | Flag any site that has been decommissioned, relocated, or renumbered since this was compiled. |

Compiled 2026-09-08 for CAP V-SAR (training simulator project) reference use. Not an official Civil Air Patrol document.